# Electronics and Digital Circuits Project

Module Title: Electronics and Digital Circuits

Module Code: 25CSCI10C

Module Leader: Assoc. Prof. Khaled M. Abdelgaber

Semester: One

University: The British University in Egypt (BUE)

Project Title: Automatic Motion Sensor Switch using CD4017 Decade Counter and E18-D80NK IR Proximity Sensor

Team (A14T4) Members:

| ID | Student Name | E-mail (BUE E-mail) |
| --- | --- | --- |
| 251551 | Youssef Wael Fawzy | youssef251551@bue.edu.eg |
| 257403 | Ziad Mostafa Gaber | ziad257403@bue.edu.eg |
| 257678 | Mahmoud Samir Moustafa | mahmoud257678@bue.edu.eg |
| 258948 | Ziad Ayman | ziad258948@bue.edu.eg |
| 244144 | Ahmed Ali Abdelmotal | ahmed244144@bue.edu.eg |

TABLE OF CONTENTS

Introduction:	2

Description of circuit:	2

Objectives:	3

Circuit Diagram:	3

Theoretical analysis	3

Data sheet:	7

STIMULATION RESULTS:	9

Proteus :	9

Hardware :	9

List of Components:	9

Conclusion:	9

# Introduction:

With the rapid growth of automation and automatic systems, there is an increasing demand for intelligent control systems that can improve everyday life in specs like efficiency and safety. Having a motion-activated switching system represents a practical and cost-effective approach.

Motion sensing systems are widely used in applications such as automatic lighting, security systems, smart home technologies, and smart doors. Traditional motion sensor control systems turn on and off when motion is detected. However, some limitations such unnecessary switching or if it is required to be kept on (the lamp) always. To overcome these issues, a manual switch is used to open and close the lamp.

Description of circuit:
This project implements an automatic motion-activated switching system that controls appliances (such as lights) based on motion detection. The system uses an E18-D80NK infrared proximity sensor to detect motion, and a CD4017 decade counter IC toggle the output state, creating a smart ON/OFF control mechanism.

Circuit Operation: The IR proximity sensor continuously emits infrared radiation. When an object or person enters the detection range, the reflected infrared signal is detected by the sensor, causing its output to go LOW. This triggers a BC557 PNP transistor, which sends a clock pulse to the CD4017 decade counter IC. The CD4017 toggles its output state (Pin-2) between HIGH and LOW with each motion detection. When Pin-2 is HIGH, it activates a BC547 NPN transistor, which energizes a 5V relay to turn ON the connected AC load. The next motion detection triggers another clock pulse, returning Pin-2 to LOW and turning OFF the load.

# Objectives:

The objective of this project is to design an automatic switch that closes and opens the light by toggling using an IR sensor that detects the motion, and this will provide energy efficiency and reduce time.

# Circuit Diagram:

# Theoretical analysis

The system consists of the following functional blocks:

IR Proximity Sensor Module (E18-D80NK) – detects motion via reflected infrared light.

Signal Conditioning Block – inverts and shapes the sensor output using a PNP transistor (Q1) and passive components.

Control Logic Block – implemented using the CD4017 decade counter to latch the activation state.

Relay Driver & Load Block – uses an NPN transistor (Q2) to energize a relay, which switches the lamp/buzzer.

Manual Reset Interface – toggle switch to reset the counter and deactivate the output.

Power Supply – 9V battery with decoupling capacitors for stability.

1. IR Proximity Sensor (E18-D80NK)

• Working Principle:

• The sensor emits infrared light and detects its reflection from nearby objects.

• When an object is within range (~3–80 cm, depending on model), reflected IR activates the internal phototransistor, causing the output pin to go LOW.

• In the absence of an object, the output remains HIGH (pulled up internally).

• This digital output is used to trigger downstream circuitry without requiring analog processing.

2. Signal Conditioning (Q1 – BC557 PNP Transistor)

• Purpose: Invert and interface the sensor signal to the CD4017 clock input.

• Operation:

• When the IR sensor output is LOW (motion detected), Q1’s base is pulled low → Q1 conducts, pulling the CLK line LOW.

• When no motion, sensor output is HIGH → Q1 cuts off, allowing CLK to return HIGH via pull-up resistor R1.

• This creates a clean falling-to-rising transition at the CD4017 clock input upon motion detection.

• Red LED (D1) lights up during detection, providing visual feedback.

3. Control Logic – CD4017 Decade Counter

• Function: Acts as a state machine to latch the "ON" command after a single trigger.

• Configuration:

• Clock Input (Pin 14): Receives pulses from Q1 via an RC network (R1, C1).

• Reset (Pin 15): Connected to a manual toggle switch for user-initiated reset.

• Clock Inhibit (Pin 13): Grounded to enable counting.

• Output Used: Q2 (Pin 2) – only this output is wired to the relay driver.

• Operation Modes:

State 0 (Q0 HIGH): System idle – relay OFF.

State 1 (Q1 HIGH): Transitional – relay still OFF.

State 2 (Q2 HIGH): Latched ON – relay activated, lamp ON.

The counter stays in State 2 until manually reset, providing memory-like behavior without a microcontroller.

4. Relay Driver & Load (Q2 – BC547, Relay RL1, Lamp L1)

• Function: Amplify the CD4017’s weak output to switch higher-power loads.

• Operation:

• When CD4017 Q2 goes HIGH, it forward-biases the base of NPN transistor Q2.

• Q2 saturates, allowing current to flow through the relay coil.

• Relay (RL1) closes its Normally Open (NO) contact, connecting the 6V lamp (L1) to the 9V supply.

• Flyback Diode (D3 – 1N4007): Suppresses voltage spikes from the relay coil’s inductance, protecting Q2.

• Green LED (D2) + R5: Indicates when the load is active.

5. Manual Reset Mechanism

• A latching toggle switch connects Vcc to the Master Reset (MR) pin of the CD4017.

• When activated, MR goes HIGH, forcing the counter back to Q0.

• This de-energizes the relay, turning OFF the lamp — allowing user control independent of motion detection.

6. Power Supply & Stability

• 9V Battery: Powers both logic (CD4017) and load (via relay).

• Decoupling Capacitors:

C3 (100µF): Stabilizes main supply.

C4 (100µF): Local filtering for CD4017 to prevent glitches during switching.

• Note: The 5V relay and 6V lamp are operated from 9V, which may cause overheating or reduced lifespan—ideally, voltage regulation or series of resistors should be used.

Overall Circuit Operation:

Standby Mode:

No motion → IR sensor output = HIGH → Q1 OFF → CD4017 remains in Q0 → Relay OFF → Lamp OFF.

Motion Detection:

Object detected → IR sensor output = LOW → Q1 turns ON → Clock pulse sent to CD4017.

CD4017 advances: Q0 → Q1 → Q2.

At Q2, output goes HIGH → Q2 (transistor) switches ON → Relay energizes → Lamp turns ON.

Red LED lights during detection; Green LED lights while lamp is on.

Latched State:

Even after motion ceases, CD4017 holds Q2 HIGH → Lamp stays ON.

Manual Deactivation:

User flips toggle switch → MR = HIGH → CD4017 resets to Q0 → Relay de-energizes → Lamp OFF.

Applications:

• Automatic lighting systems (e.g., closet lights that stay on after entry).

• Security alarms (motion triggers a persistent alert until acknowledged).

• Energy-saving switches with "stay-on" functionality.

• Educational projects demonstrating digital latching without microcontrollers.

# Data sheet:

CD4017

LED (light- emitting diode):

E18-D80NK IR Obstacle Avoidance Proximity Sensor:

5V Relay

bc547 transistor

bc557 transistor

Resistors

Capacitors

Diode

# STIMULATION RESULTS:

Simulation Results

1. Initial State (No Motion Detected)

IR Sensor Output: HIGH (5V logic level)

Q1 (BC557): OFF → CLK line pulled HIGH via R1 (10kΩ)

CD4017 State: Reset to Q0 (Pin 3 = HIGH; all others LOW)

Q2 (BC547): OFF → Relay coil current = 0A

Lamp (L1): OFF (0V across terminals)

LEDs: D1 (Red) OFF, D2 (Green) OFF

Circuits are idle and consume minimal power.

2. Motion Detected (First Trigger)

IR Sensor Output: Pulled LOW (0V) for ~100ms (simulating object passing)

Q1 (BC557): Turns ON → CLK line pulled LOW

RC Network (R1/C1): Generates a clean falling edge → rising edge at CLK pin triggers CD4017

CD4017 Advances: Q0 → Q1 → Q2

Q2 (Pin 2) goes HIGH (≈5V)

Q2 (BC547): Turns ON → Base current ≈ 2mA (via R4=220Ω) → Collector current ≈ 40mA (saturating relay)

Relay RL1: Coil energized (current ≈ 40mA), NO contact closes

Lamp L1: Voltage rises to 8.7V (slight drop due to relay resistance) → lamp illuminates

LEDs:

D1 (Red) lights up during trigger (motion detected)

D2 (Green) lights up and stays on (lamp active)

Motion successfully triggers latched ON state.

3. Post-Trigger State (Motion Gone, Lamp Still On)

IR Sensor Output: Returns to HIGH → Q1 turns OFF → CLK returns to HIGH

CD4017: Remains in Q2 state (latched) → Q2 output stays HIGH

Q2 (Transistor): Remains ON → Relay stays energized → Lamp stays ON

LEDs: D1 OFF (no motion), D2 ON (lamp active)

Circuit maintains “ON” state even after motion ends — true latching behavior.

4. Manual Reset (Toggle Switch Activated)

Toggle Switch: Closed → MR (Pin 15) pulled HIGH

CD4017: Immediately resets to Q0 → Q2 output goes LOW

Q2 (Transistor): Turns OFF → Relay de-energizes → Lamp turns OFF

LEDs: D2 (Green) turns OFF

Users can manually override and turn off the lamp.

5. Second Motion Detection (After Reset)

IR Sensor Output: Pulled LOW again

CD4017 Advances: Q0 → Q1 → Q2 → Lamp turns ON again

Circuit responds reliably to repeated triggers.

Proteus:

Hardware :

# List of Components:

| Component | Function/Type |
| --- | --- |
| Integrated Circuits | - CD4017 |
| Transistors | - · BC557 PNP Transistor - 1-unit BC547 NPN Transistor - 1 unit |
| Resistors | 220Ω, 0.25W resistors - 3 units 4.7kΩ, 0.25W resistor - 1-unit 10kΩ, 0.25W resistor - 1 unit |
| Capacitors | · 100µF, 25V electrolytic capacitor - 1-unit 1000µF, 25V electrolytic capacitor - 1 unit |
| Sensors | E18-D80NK IR Proximity Sensor Module - 1 unit |
| Diodes & LEDs: | · 1N4007 Rectifier Diode - 1-unit 5mm LEDs (indicator lights) - 2 unit |
| Relay & switching | · 5V SPDT (Single Pole Double Throw) Relay - 1 unit Latched toggle switches - 1 units |
| Other Components | 6V Lamp Jumper Wires |

Conclusion:

The project, an automatic Moton-activated switching system, was successfully designed and analyzed. The system detects motion and toggles the state of load between ON and OFF with each detection, demonstrating reliable and intelligent control behavior. Overall automatic systems limit regular and time-wasting tasks and increase efficiency. A project like this helps in automatic lighting that saves time for the user, like the user doesn't search for the light switch to open it manually.

Some problems faced while doing this project:

- Proteus

  - Not having clear schematic, or clear reference made the proteus implantation a little hard and time consuming

- Implantation hardware:

  - The most challenging thing was that the IR sensor is not working in some situations, and the reason was that the IR sensor detects black objects more.

- Not finding the exact value capacitors in real life as mentioned in the reference

- Find the right Sensor to work with.

Reference : 
Motion Sensor Switch Using IR Proximity Sensor | CD4017 IC Projects : 11 Steps - Instructables